Reg. No.									                      CODE: CORENT

PH. D COURSEWORK EXAMINATION, MAY 2023

SUBJECT: NANOTECHNOLOGY

CORE PAPER

Time: 03 hours						                                         Maximum Marks: 50

Instructions: Answer 5 Questions

(5+5)

- a) Briefly mention various covalent and non-covalent surface functionalization strategies for carbon                   nanotube and graphene. Explain the importance of functionalization.

b) Explain how dimensionality of the materials plays a crucial role in catalysis such as water splitting, oxygen reduction reaction and supercapacitors. Mention advantage of making nanocomposites of various layered materials for catalysis applications.

- a)   Elucidate Jablonski diagram to represent all photo physical properties.                                 (7.5+2.5)

- Discuss the reason for high and low quantum yield with examples?

- a)  Define toxicity and write a note on size and shape-dependent toxicity of nanomaterials       (4+3+3)

- Write a note on bioinert and bioactive materials

- Discuss in brief the ways to control the toxicity of nanomaterials

(5+5)

- a)   The wavelength of the X-rays is given as 0.071 nm. It is diffracted by a plane of salt with a lattice constant of 0.28 nm. Calculate the glancing angle for the second-order diffraction. Assume the salt plane value is 110, and the given salt is FCC structure

- What is the Raman Effect?  Write down three basic applications of Raman spectroscopy

(5+5)

- a)  Explain the principle of Dip Pen Nanolithography (DPN).  Write down the advantages of DPN over photo lithography

- Which deposition process is most likely to lead layer growth ? Explain the mechanism of chemical vapor deposition (CVD) with schematics. Write down three basic differences between CVD and PVD

(4+3+3)

- a) As the nanoparticles (NP) travel through the human body, what is the environment like? Explain at least 3 conditions or factors in the human body that might cause the NP to degrade.

- Why is it beneficial to be able to have specific targets for the nanoparticles?

- What are the factors to be considered before you use nanoàprticles for in vivo biomedical applications?

**********